# Decision 18:  Single- or Double-Stage Bunch Compressor

## What are the options?

There are two basic options for compression of the bunch extracted from the damping ring:

1.	Perform the compression in a single stage

2.	Perform the compression in 2 stages, with acceleration between the two stages.

Other multi-stage compressors have been considered but any advantages do not offset the added cost and complexity. A 180º turn has been proposed between the damping ring and linac, but this would not be part of the compressor.

## Pros and cons of Bunch Compressor Designs

Cost:  Favors 1 over 2.  Performing the compression in 2 stages requires 2 wigglers, each of which must be fairly long in order to limit emittance growth from synchrotron radiation.  The two stage system also requires additional non-accelerating RF (to provide the energy-position correlation needed for compression) and additional matching sections.  The exact additional costs are somewhat design-dependent, but as a rough guide the two-stage could require as much as 0.5 km of additional non-accelerating tunnel length per side as compared to the single stage, or about 1 km additional length over the full ILC site.

Parameters:  Favors 2 over 1.  The current proposed operating plane of the ILC includes parameter sets with IP bunch lengths as small as 150 µm RMS.  Assuming a bunch length of 6 mm in the damping ring, compressing to 150 µm in a single stage would lead to an RMS energy spread at the exit of the compressor of about 10%, which is unacceptable.  Thus, a single stage bunch compressor is ruled out unless the parameter sets with IP bunch lengths of under 300 µm are eliminated.

Luminosity:  Favors 2 over 1.  Even in the nominal parameter set (compression from 6 mm to 300 µm), the two-stage bunch compressor leads to looser alignment tolerances for quadrupoles, BPMs, and RF cavities than the single-stage design.  In addition, the two-stage design would be able to accept a bunch as long as 9 mm from the damping ring and still achieve a final length of 300 µm with acceptable emittance growth in the Low Emittance Transport. This would not be possible in the case of the single-stage compressor.

Extra R&D needed before making a decision

## No additional R & D is needed before making the principal decision (single vs double stage compressor).  At this time there are several two-stage BC designs with different properties, and some additional effort is required to compare their costs and tolerances.  This effort can be concluded by the end of 2005.

## Recommendation for the BCD

Use a two-stage bunch compressor.